Global requirements for a feasible experiment:

- Only fixed target experiments are possible. It is not possible to bring two beams into collision

- It is also possible to use radioactive materials or large (more than 1 kg) amounts of flammable materials as a target.

- The experimental area has a size of 1x1 m. All detectors must be placed inside that area

- It must be possible to perform the experiment in less than 10 days (maximum duration of the beam time). Beam is available for 12 hours per day.

- Access to the experimental area is possible on weekdays but limited to 2-3 accesses per day.

- The purpose of the experiment can be the observation of an effect in particle physics, the test of a new detector or creative combination of art and science.

- The experiment must be at a level that allows very skilled high school students to at least roughly understand its details and the physics formulas that it depends on.

- CERN prefers experiments that provide an immediate feedback via electronic or optical sensors. The students will analyze the data and decide in real-time on the next steps to take (e.g. modifications of the setup or the beam parameters). Experiments that are based on the irradiation of samples and the off-line analysis of these samples in remote laboratories are permitted but discouraged.

Parameters of the beam:

- The beam is provided by the CLEAR facility of CERN. It is a pure electron beam.

- The beam’s momentum ranges from 50 to 220 MeV/c and can be selected by the user

- Bunch charge: 0.01 - 1.5 nC

- Normalized emittances: 3 um for 0.05 nC per bunch, 20 um for 0.4 nC per bunch (in both planes)

- Bunch length: ~100 um - 1.2 mm

- Relative energy spread: < 0.2 % rms (< 1 MeV FWHM)

- Repetition rate: 0.8 - 10 Hz

- Number of micro-bunches in train: 1 - 150

- Micro-bunch spacing: 1.5 or 3.0 GHz

- It is possible to attenuate the beam such that only one electron is delivered (on average) per bunch

List of available detectors:

- Scintillators

- Lead glass calorimeters

- Neutron dose meters (WENDI)

- YAG screens and CCD/CMOS cameras

- TimePix3 and Katherine read-out system

Details about the detectors:

- Scintillators

  - Number of detectors available: 3

- Size: 10 x 10 cm

- Material: EJ200

- Read-out: PMT

- Efficiency: ~95%

- Limitation: Can only be used with a low intensity beam

- Lead glass calorimeters

  - Number of detectors available: 3

  - Cross section: 10 x 10 cm

  - Length: 37 cm

  - Sensor: PMT

  - Material: lead glass

  - Resolution: σE / E = 0.02% + 6.3% / √E

  - Read out with: QDC

  - Limitation: Can only be used with a low intensity beam

- Neutron dose meters (WENDI)

  - Number of detectors: 1

  - Measured quantity: neutron dose (sievert)

- YAG screens and CCD/CMOS cameras

  - Beam interacts with YAG screen and produces visible light

  - CCD/CMOS camera looks at the YAG screen and records the optical effects

- TimePix3 and Katherine box

  - The TimePix3 is a pixel sensor (256 × 256 pixels with a 55 µm pitch). It can detect the time of arrival of a particle. In addition, a (Time over Threshold measurement for charge / energy deposition is possible. For particles with an energy of 3-20 keV it works like a calorimeter.

- Detector prototypes

  - It is possible to install detector prototypes in the experimental area and to test their reaction to the beam. These detectors must not require very expensive materials (budget limit: USD 2000) for their construction.

Details about the data acquisition system:

- For creating a trigger signal all common types of NIM modules are available. This includes discriminators and coincidences. The NIM electronics modules have a time resolution of at least 5 nano seconds.

- For the digitization of analog signals, charge digitizer (QDC) and time digitizer (TDC) are available. There are also modules that can count signals. There are no ADCs available.

- The data acquisition system can record up to 2000 events per second